Model 4160 Connector, BNC (Female), Bulkhead

FEATURES:

•    Connector meets the interface requirements of MIL-STD-348

MATERIALS:

Body and Fittings – Brass, Tarnish Resistant Plating

Center Contact – Gold Plated Beryllium Copper

Dielectric: PTFE

Marking: “Pomona 4160”

RATINGS:

Operating Voltage: Shield/Earth: 30VAC/60VDC Max.

Inner Conductor/Shield: 500VRMS Max.

Operating Temperature: -65ºC to +165ºC

Nominal Impedance: 50 Ohms

ORDERING INFORMATION: Model 4160

Parts are packaged one per bag, unassembled.

All dimensions are in inches. Tolerances (except noted): xx = ±.02” (,51 mm), .xxx = ± .005” (,127 mm).

All specifications are to the latest revisions. Specifications are subject to change without notice.

Registered trademarks are the property of their respective companies.  Made in USA

5/30/02 SY/EH/LS

S:\Engineering\Release\DataSheets\FlukeDataSheet\d4160_1_02.doc 	 	    Page 1 of 1